Worksheet IP.2 – Image Transforms

Question 1. Given a 3x3 gray-scale image:

50  100  150 200  250  100 150  10  255

Apply the following image intensity transformation functions to the image:

- Negative Transformation: Inew=255–Iold

- Log Transformation: Inew=c⋅log1+Iold where c = 45

- Power-Law (Gamma) Transformation: Inew=c⋅Ioldγ where c = 1 and γ=0.5

Question 2. Consider a 5x5 gray-scale image:

102030405060708090100110120130140150160170180190200210220230240250

- Create the homography matrix for the following operations:

  - Rotate the image by 45 degrees. (counter clockwise).

  - Scale the image by a factor of 2.

  - Translate the image by (3, 4) pixels.

- Apply the transformation to the pixel originally at position (2, 3) with value 130. What will be the new value and location of this pixel?

Question 3. Given a 30x30 2-bit image with the following histogram counts, show the mapped values using histogram equalization.

- Intensity 0: 80 pixels

- Intensity 1: 800 pixels

- Intensity 2: 0 pixels

- Intensity 3: 200 pixels