# Worksheet IP.4 – Image Processing in Fourier Domain

Question 1. A 2D continuous function has maximum frequency components of 60 Hz in the horizontal direction and 45 Hz in the vertical direction. You sample the function at 100 samples per second in the horizontal direction and 80 samples per second in the vertical direction.

- Determine whether aliasing will occur.

- If aliasing occurs, suggest a new sampling rate to avoid it.

Question 2. You are filtering a 2048×2048 image using a 201×201 non-separable kernel. Estimate the number of operations required for:

- Direct spatial filtering

- Frequency domain filtering using FFT

Question 3. You want to convolve two 2D discrete signals of size 128×128 and 64×64 using Circular DFT. What is the minimum size of zero-padding required to avoid wraparound error?

Question 4. Design a Butterworth lowpass filter of order 2 with cutoff frequency D0=50 for a 256×256 image. Compute the filter value at point (u,v) = (60, 60).