Worksheet IP.1 – Image Fundamentals- Solutions

Question 1. An object of height 2 meters is placed 5 meters away from the eye. The image distance di from the lens to the retina is approximately 17 mm.

- Calculate the height of the image formed on the retina. Is the entire retinal image formed on the fovea, given that the fovea is about 1.5 mm in diameter?

- Calculate the focal length of the lens.

Solution.

a) Calculate the size of image

Using the magnification formula: M=hiho=dido

Where:

- hi is the height of the image

- ho is the height of the object (2 m)

- di is the image distance (17 mm or 0.017 m)

- do is the object distance (5 m)

Rearranging to find hi: hi=ho×dido

Substituting the values:

hi=2m×0.017m5 m= 0.0068 m=6.8 mm

The fovea is about 1.5 mm in diameter. Since the height of the image (6.8 mm) is greater than the diameter of the fovea (1.5 mm), the entire retinal image will not be on the fovea.

b) Calculate the focal length of the lens

Using the pinhole optics formula: 1zo+1zi=1f

Where:

- zo is the distance of the object from the lens (approximately the same as distance form the eye)

- zi is the distance of the image from the lens

- f is the focal length of the lens

Therefore:

f=11zo+1zi=115000+117=16.94mm

Note that when the object is far 1zo is negligibly small, and therefore f≈zi.

Question 2. Consider a binary digital image represented by the following 5x5 pixel matrix:

- Identify the 4-neighborhood and 8-neighborhood of the pixel at position (2, 2), assuming 1-based indexing, i.e., the top-left corner of the matrix is at position (1, 1).

- Determine if the pixel at position (2, 2) is 4-adjacent, 8-adjacent, or m-adjacent to the pixels at

  - position (3, 2),

  - position (1, 1),

  - position (3, 3).

Solution.

a)  The 4-neighborhood and 8-neighborhood of the pixel at position (2, 2)

The pixel at position (2, 2) has a value of 1.

4-neighborhood: The 4-neighbors are the pixels directly adjacent to (2, 2) in the horizontal and vertical directions.

The 4-neighbors of (2, 2) are:

- (1, 2) → 0

- (3, 2) → 1

- (2, 1) → 0

- (2, 3) → 1

8-neighborhood: The 8-neighbors are the pixels directly adjacent to (2, 2) in the horizontal, vertical, and diagonal directions.

The 8-neighbors of (2, 2) are:

- (1, 1) → 1

- (1, 2) → 0

- (1, 3) → 0

- (2, 1) → 0

- (2, 3) → 1

- (3, 1) → 0

- (3, 2) → 1

- (3, 3) → 0

b) Determine if the pixel at position (2, 2) is 4-adjacent, 8-adjacent, or m-adjacent to the pixels at (i) position (3, 2), (ii) position (1, 1), and (iii) position (3, 3).

- Pixel at position (3,2)

First, both pixels have values from V= {1}:

- The pixel at (2, 2) has a value of 1.

- The pixel at (3, 2) has a value of 1.

4-adjacency: Two pixels are 4-adjacent if they are neighbors in the horizontal or vertical direction and have values from V.

Since the values are the same and they are in 4-neighboorhood of each other, they are 4-adjacent.

8-adjacency: Two pixels are 8-adjacent if they are neighbors in the horizontal, vertical, or diagonal direction and have values from V.

Similarly, the two pixels are 8-adjacent.

m-adjacency: Two pixels are m-adjacent if they are 4-adjacent or diagonal neighbors that do not have common 4-neighbors with values from V.

Since they are 4-adjacent, they are also m-adjacent.

- Pixel at position (1,1)

First, both pixels have values from V= {1}:

- The pixel at (2, 2) has a value of 1.

- The pixel at (1, 1) has a value of 1.

4-adjacency: This pixel is not in 4-neighboorhood of (2,2), and therefore, they are not 4-adjacent.

8-adjacency: This pixel is in 8-neighboorhood of (2,2) and has value from V. Therefore, these two pixels are 8-adjacent.

m-adjacency: This pixel is a diagonal neighbor of (2,2). The common 4-neighbors are (1,2) and (2,1), none of which have with values from V. Therefore, these two pixels are m-adjacent.

- Pixel at position (3,3)

These pixels do not have the first condition of having values from V= {1}:

- The pixel at (2, 2) has a value of 1.

- The pixel at (3, 3) has a value of 0.

Therefore, these two pixels are not 4-adjacent, 8-adjacent, or m-adjacent.

Question 3. Consider two pixels in a digital image with coordinates P1(3, 4) and P2(7, 1). Calculate the following distances between these two pixels:

- Euclidean distance

- City-block (Manhattan) distance

- Chessboard (Chebyshev) distance

- Minkowski distance with p = 3

Solution.

a)  Euclidean distance

The Euclidean distance between two points (x1, y1) and (x2, y2) is given by:

dE=x2–x12+y2-y12

Substituting the coordinates:

dE=7-32+1-42=42+-32dE=16+9=25=5

b)  City-block (Manhattan) distance

The City-block distance between two points (x1, y1) and (x2, y2) is given by:

dM=x2–x1+y2-y1=7-3+1-4=4+3=7

c) Chessboard (Chebyshev) distance

The Chessboard distance between two points (x1, y1) and (x2, y2) is given by:

dC=maxx2–x1,y2-y1=max7-3,1-4=max4,3=4

d) Minkowski distance with p = 3

The Minkowski distance between two points (x1, y1) and (x2, y2) with parameter p is given by:

dMp=x2-x1p+y2-y1p1/p

Substituting the coordinates and p = 3:

dM3=7-33+1-431/3=43+-331/3=64+-271/3=371/3≈3.33