# Worksheet IP.4 – Image Processing in Fourier Domain – Solutions

Question 1. A 2D continuous function has maximum frequency components of 60 Hz in the horizontal direction and 45 Hz in the vertical direction. You sample the function at 100 samples per second in the horizontal direction and 80 samples per second in the vertical direction.

- Determine whether aliasing will occur.

- If aliasing occurs, suggest a new sampling rate to avoid it.

Solution.

- Nyquist Rates:

  - Horizontal:fsx=2×60=120Hz

  - Vertical:  fsy=2×45=90 Hz

- Given Sampling Rates:

  - Horizontal: 100 Hz → Below Nyquist → Aliasing occurs

  - Vertical: 80 Hz → Below Nyquist → Aliasing occurs

- Suggested Rates:

  - Horizontal: >120 Hz

  - Vertical: >90 Hz

Question 2. You are filtering a 2048×2048 image using a 201×201 non-separable kernel. Estimate the number of operations required for:

- Direct spatial filtering

- Frequency domain filtering using FFT

Solution.

- Direct Spatial Filtering:

Number of operations =MNmn=20482×2012=1.695×1011 operations

- FFT Filtering:

Number of operations =2MNlog2MN=2×20482×log220482=2×20482×22≈1.84×108 operations

Therefore, the filtering in frequency domain is approximately 918 times faster.

Question 3. You want to convolve two 2D discrete signals of size 128×128 and 64×64 using DFT. What is the minimum size of zero-padding required to avoid wraparound error?

Solution.

To avoid wraparound error, pad both signals to size P×Q where:

P = M + m - 1 = 128 + 64 - 1 = 191

Q = N + n - 1 = 128 + 64 - 1 = 191

Therefore, both signals must be zero padded at a minimum to size 191×191 to avoid wraparound error.

Question 4. Design a Butterworth lowpass filter of order 2 with cutoff frequency D0=50 for a 256×256 image. Compute the filter value at point (u,v) = (60, 60).

Solution:

When filtering an image of size M × N in the frequency domain, we zero pad it to size 2M × 2N before applying the DFT. The DFT assumes periodicity, so without padding, convolution in the frequency domain results in circular convolution, which causes wraparound errors. Padding ensures that the linear convolution result fits within the padded array, avoiding overlap.

If you're applying a filter (e.g., Butterworth) in the frequency domain, it must be the same size as the padded image – which is the same size in the frequency domain.

Therefore, if the image is padded to 2M × 2N, the filter must also be defined over a 2M × 2N grid. Thus, the filter size is 512 × 512.

The Butterworth filter is defined as:

Hu,v=11+Du,vD02n,    where  Du,v=u-M/22+ v-N/22

Distance from center:

Du,v=60-2562+60-2562=-1962+-1962≈277.19

Butterworth filter value at (60,60):

Hu,v=11+Du,vD02n=11+277.19504≈11+944.50≈0.0011